# LAB # 10

Arrays in RISC-V using Assembly

# Objective:

- To understand and implement basic array operations in Assembly using RISC-V architecture.

# System Module (Hardware/Software)

- Visual Studio Code (Venus Simulator)

# Theoretical Background:

An array is a collection of elements, typically of the same data type, stored in contiguous memory locations. In programming, arrays allow us to store and manage multiple values under a single name, accessed using an index.

Consider an array of 5 elements, each accessed using its index from 0 to 4. If we want to double the contents of elements stored at index 0 and 1, then we can do so following the instructions below:

// C Code

int array[5];

array[0] = array[0] * 2;

array[1] = array[1] * 2;

#RISC-V assembly code

#s0 = base address of array

li s0,0x00000780	#initialize base address

lw t0, 0(s0)		#t0 = array[0]

slli t0, t0, 1		#t0 = t0*2

sw t0, 0(s0)		# array[0] = array[0] * 2

lw t0, 4(s0)		# t0 = array[1]

slli t0, t0, 1		#t0 = t0*2

sw t0, 4(s0)		# array[0] = array[0] * 2 stored on memory

Task 1: Accessing array elements using loops

Consider the C-code for accessing an array of 1000 elements:

int array[1000];

int i;

for(i=0, i<1000, i++)

array[i] = array[i]*8;

Now converting the above C-code in assembly:

addi s0, zero, 00000400

li s1, 0

addi t2, zero, 1000

loop:

bge s1, t2, done

slli t0, s1, 2

add t0, t0, s0

lw t1, 0(t0)

...

...

addi s1, s1, 1

j loop

done:

nop

Task 2: Sum of elements in an array

Write an assembly code to add through all the numbers stored in an array (at least 10 elements long). First, store random values in memory in contiguous locations.

Hint: load indices of first and last elements in temporary registers, multiply them by 4 to get equivalent addresses of their locations (base addr + i*4), then swap them.

# Learning Outcomes:

Upon successful completion of the lab, students will be able to: LO1: Access and manipulate array elements using pointers.