# LAB # 7

Load & Store Instruction Assembly Code

# Objective:

- Get Familiar with Load & Store Instruction Assembly Code

# System Module (Hardware/Software)

- Venus Simulator

# Theoretical Background:

## Introduction:

RV32I is a load-store architecture, where only load and store instructions access memory and arithmetic instructions only operate on CPU registers. RV32I provides a 32-bit user address space that is byte-addressed and little-endian. The execution environment will define what portions of the address space are legal to access. Loads with a destination of x0 must still raise any exceptions and action any other side effects even though the load value is discarded.

## Load Instructions:

Load instructions are used to move data from memory to registers (before operation). Loads are encoded in the I-type format. The effective byte address is obtained by adding register rs1 to the sign-extended 12-bit offset. Loads copy a value from memory to register rd. The assembly representation for load instructions are:

## lw (destination_register), (offset)(source_register) or

lw (rd), offset(rs1)

There are many types of load instructions. RISC-V support the following types

| Types | Symbol |
| --- | --- |
| Load-Byte (signed) | LB |
| Load-Byte (unsigned) | LBU |
| Load-Halfword (signed) | LH |
| Load-Halfword (unsigned) | LHU |
| Load-Word | LW |

The LW instruction loads a 32-bit value from memory into rd. LH loads a 16-bit value from memory, then sign-extends to 32-bits before storing in rd. LHU loads a 16-bit value from memory but then zero extends to 32-bits before storing in rd. LB and LBU are defined analogously for 8-bit values.

The instruction format of Load instructions is shown below:

## Store Instructions:

Store instructions are used to move data from registers to memory (after operation). Stores are encoded in the S-type format. The effective byte address is obtained by adding register rs1 to the sign-extended 12-bit offset. Stores copy the value in register rs2 to memory. The assembly representation for store instructions are:

## sw (source_register_2), (offset)(source_register_1) or

sw (rs2), offset(rs1)

There are many types of store instructions. RISC-V support the following types

| Types | Symbol |
| --- | --- |
| Store-Byte (signed) | SB |
| Store-Halfword (signed) | SH |
| Store-Word | SW |

The SW instruction stores a 32-bit value from the low bits of register rs2 to memory. SH stores a 16-bit value from the low bits of register rs2 to memory. SB stores a 8-bit value from the low bits of register rs2 to memory.

The instruction format of Store instructions is shown below:

# In-Lab Tasks:

## Lab Exercise 1:

Run the below assembly code on Venus Simulator and observe the relevant registers and memory.

li s0, 0x12345678 # Data to be stored

li s1, 0x00000020 # memory address sb s0, 0x0(s1)

sh s0, 0x4(s1) sw s0, 0x8(s1)

## Lab Exercise 2:

Run the below assembly code on Venus Simulator lb t0, 0x0(x0)

lbu t1, 0x4(x0) lh t2, 0x8(x0) lhu s0, 0xC(x0) lw s1, 0x10(x0)

# Post Lab Tasks:

- Write down a simple assembly program to add, and subtract two integer numbers and store their result into different memory locations. Simulate the code on Venus.

- Write down a simple assembly program to load the contents from memory into registers and perform the logical operations on them. Simulate the code on Venus.

# Learning Outcomes:

Upon successful completion of the lab, students will be able to: LO1: Run the basic assembly code on Venus Simulator.