# LAB # 8

Arithmetic and Branch Instruction Assembly Code

# Objective:

Get Familiar with Arithmetic and Branch Instruction Assembly Code

# System Module (Hardware/Software)

- Venus Simulator

# Theoretical Background:

Introduction:

Arithmetic Instructions:

Most integer computational instructions operate on XLEN bits of values held in the integer register file. Integer computational instructions are either encoded as register-immediate operations using the I-type format or as register-register operations using the R-type format. The destination is register rd for both register-immediate and register-register instructions. No integer computational instructions cause arithmetic exceptions.

The instruction format for register-immediate instruction is shown below:

Types of Register-Immediate instruction supported in RISC-V are:

| Types | Symbol |
| --- | --- |
| Add Immediate | ADDI |
| Set Less Than Immediate (signed) | SLTI |
| Set Less Than Immediate (unsigned) | SLTIU |
| And Immediate | ANDI |
| Or Immediate | ORI |
| Xor Immediate | XORI |

| Shift Left Logical Immediate | SLLI |
| --- | --- |
| Shift Right Logical Immediate | SRLI |
| Shift Right Arithmetic Immediate | SRAI |

ADDI adds the sign-extended 12-bit immediate to register rs1, SLTI (set less than immediate) places the value 1 in register rd if register rs1 is less than the sign-extended immediate when both are treated as signed numbers, else 0 is written to rd. SLTIU is similar but compares the values as unsigned numbers (i.e., the immediate is first sign-extended to XLEN bits then treated as an unsigned number). ANDI, ORI, XORI are logical operations that perform bitwise AND, OR, and XOR on register rs1 and the sign-extended 12-bit immediate and place the result in rd. The operand to be shifted is in rs1, and the shift amount is encoded in the lower 5 bits of the I-immediate field. SLLI is a logical left shift (zeros are shifted into the lower bits); SRLI is a logical right shift (zeros are shifted into the upper bits); and SRAI is an arithmetic right shift (the original sign bit is copied into the vacated upper bits).

The assembly representation for register-immediate instructions are:

addi rd, rs1, imm slti rd, rs1, imm ori rd, rs1, imm

RV32I defines several arithmetic R-type operations. All operations read the rs1 and rs2 registers as source operands and write the result into register rd. The funct7 and funct3 fields select the type of operation.

The instruction format for register-register instruction is shown below:

Types of Register-Register instruction supported in RISC-V are:

| Types | Symbol |
| --- | --- |
| Addition | ADD |
| Subtraction | SUB |
| Set Less Than (signed) | SLT |
| Set Less Than (unsigned) | SLTU |
| Logical And | AND |
| Logical Or | OR |

| Logical Xor | XOR |
| --- | --- |
| Shift Left Logical | SLL |
| Shift Right Logical | SRL |
| Shift Right Arithmetic | SRA |

ADD and SUB perform addition and subtraction respectively. Overflows are ignored and the low XLEN bits of results are written to the destination. SLT and SLTU perform signed and unsigned compares respectively, writing 1 to rd if rs1 < rs2, 0 otherwise. AND, OR, and XOR perform bitwise logical operations. SLL, SRL, and SRA perform logical left, logical right, and arithmetic right shifts on the value in register rs1 by the shift amount held in the lower 5 bits of register rs2.

The assembly representation for register-register instructions are:

add rd, rs1, rs2 and rd, rs1, rs2 sub, rd, rs1, rs2

Branch Instructions:

All branch instructions use the B-type instruction format. The 12-bit B-immediate encodes signed offsets in multiples of 2, and is added to the current pc to give the target address. The conditional branch range is ±4 KiB.

The instruction format for branch instruction is shown below:

Types of Branch instruction supported in RISC-V are:

| Types | Symbol |
| --- | --- |
| Branch if Equal | BEQ |
| Branch if Not Equal | BNE |
| Branch Less Than (signed) | BLT |
| Branch Less Than (unsigned) | BLTU |
| Branch Greater Than (signed) | BGE |
| Branch Greater Than (unsigned) | BGEU |

Branch instructions compare two registers. BEQ and BNE take the branch if registers rs1 and

rs2 are equal or unequal respectively. BLT and BLTU take the branch if rs1 is less than rs2, using signed and unsigned comparison respectively. BGE and BGEU take the branch if rs1 is greater than or equal to rs2, using signed and unsigned comparison respectively

Lab Exercise 1:

Run the below assembly code on Venus Simulator addi x8, x0, 0x5

ori x9, x0, 0x7 add x10, x8, x9 or x5, x9, x10 slt x6, x10, x5

Lab Exercise 2:

Run the below assembly code on Venus Simulator addi x5, x0, 0x0

addi x6, x0, 0x6 loop:

addi x5, x5, 0x1 bne x5, x6, loop

addi x7, x0, 0x10

# Procedure:

Follow the same procedure to run Venus as we did in Lab 1.

# In-Lab Tasks:

- Write a simple assembly program to load two numbers in saved registers, compare them, store the lesser one in X7 register. (Hint: use branch instructions)

- Write a simple assembly program to calculate the sum of first 10 natural numbers. Simulate the code on Venus.

- Write an assembly program to check whether a number is even or not. Hint: use ‘andi’ instruction along with branch instruction to compare the result. Modify the program in task 2 to add even numbers in the range of 1-15.

# Learning Outcomes:

Upon successful completion of the lab, students will be able to: LO1: Run the basic assembly code on Venus Simulator.