# LAB # 9

Programming with Assembly

# Objective:

- Programming in Assembly Language using RISC-V architecture.

# System Module (Hardware/Software)

- Venus Simulator

## Task 1:

Convert -31 in binary (Use 2’s Complement):

Extend it to 32b:

Write instruction to load the above number in any saved register:

Add 6 in it. Store the result in a temporary register. (Use I-type instruction)

## Task 2:

Load -1 in a saved register(say s3):

Load a hex number 0x1234ABCD in another saved register (say s4):

Perform AND operation among the above two registers (s3 and s4), save the result in a third register (say s5):

Now, perform AND between s4 and x0(zero), save the result in s6:

What did you observe?

## Task 3:

Repeat Task 2 by performing XOR operation among values loaded in saved registers.

What did you observe when:

- Any operand (data) was XORed with -1:

- Any operand was XORed with zero:

What would happen when 0x1234ABCD is ORed with 0x0000FFFF?

## Task 4:

Now load 230 = 0x40000000 and -231 = 0x80000000 in two different registers (say s6 and s7).

Perform multiplication using following syntax:

mul s8, s6, s7

mulh s9, s6, s7

Convert the result in decimal and verify the operation mathematically.

## Task 5:

Perform division for the given case using following commands:

26÷5 = 5 15

Assuming s0 = 5, s1 = 26

div s3, s1, s0

rem s4, s1, s0

## Task 6:

Load a number 0x5 in s1. Shift it towards left by 2 bits using I-type and R-type instructions:

Store the result in t0.

Now shift the number 0x80000000 (stored in s2) 4 bits towards right using immediate type instruction once logical and second arithmetically

What changes in the result did you observe:

## Task 7:

Run the following code in Venus simulator:

addi s0, zero, 4

addi s1, zero, 1

slli s1, s1, 2

beq s0, s1, target

addi s1, s1, 1

sub s1, s1, s0

target: add s1, s1, s0

nop

Now run the following code, observe the difference in branch execution when compared with above set of instructions.

addi s0, zero, 4

addi s1, zero, 1

slli s1, s1, 2

bne s0, s1, target

addi s1, s1, 1

sub s1, s1, s0

target: add s1, s1, s0

nop

Now try any piece of code by using unconditional jump command

Here’s the syntax: j label

FYI: It skips the lines of code in sequence and directly jumps to the label/branch name (unconditionally) mentioned immediately after it.

## Task 8 (Conditional Statements):

- Complete the given code for the following pseudocode:

Check if i = j:

Yes? c = a+b

#s0 = a, s1 = b, s2 = c, s3 = i, s4 = j

li s1, 8

li s2, 2

li s3, 1

li s4, 1

beq …

…

…

nop

Try the same using bne now.

## Task 8(b):

Try the same Task 8(a) by adding an else statement as well.

Check if i = j:

Yes? c = a+b

No/Else: c = c-i

## Task 9 (Loops):

- Use while loop to calculate the power of a number whose base is 2.

2n = x

pseudocode:

n = 0 #initialization

p = 1

x = 2n.

while (p ≠ x)

p = p * 2

n = n + 1

n returns the power of that number to base 2.

Try the code below to check its execution for different values of t0

li s0, 0x1

li s1, 0x0

addi t0, zero, 128

while: beq s0, t0, done

slli s0, s0, 1

addi s1, s1, 0x1

j while

done:

nop

Task 9(b):

Run a for loop to find a series of first 10 whole numbers.

Pseudocode:

sum = 0 	#initialization

int i 		#Index i

for (i = 0, i≠10, I = i++)

sum = sum+i

Assembly Code:

# s0 = i, s1 = sum

addi s1, zero, 0

addi s0, zero, 0

addi t0, zero, 10

for: beq s0, t0, done

add s1, s1, s0

addi s0, s0, 0x1

j for

done:

…

nop

# Learning Outcomes:

Upon successful completion of the lab, students will be able to: LO1: Run the different instructions of assembly code on Venus simulator.